# 1. Introduction

## 1.1 Global ozone risk and regulatory context

Surface ozone (O3) is a secondary air pollutant formed through photochemical reactions involving nitrogen oxides (NOx), volatile organic compounds (VOCs), carbon monoxide (CO), sunlight, and meteorological conditions. Unlike primary pollutants emitted directly from sources, ambient O3 reflects the combined influence of precursor emissions, atmospheric chemistry, regional inflow, dry deposition, and boundary-layer processes. Elevated O3 is harmful to human health, vegetation, crops, and ecosystem productivity (Fleming et al., 2018; Mills et al., 2018; World Health Organization [WHO], 2021). For exposure and management assessment, maximum daily 8-h average ozone (MDA8 O3) is widely used because it captures sustained daytime exposure rather than short hourly peaks alone. The 2021 WHO global air-quality guidelines recommend an O3 daily maximum 8-h guideline of 100 µg/m³ (≈50 ppb) and a peak-season mean of daily maximum 8-h O3 of 60 µg/m³ (≈30 ppb), whereas Taiwan's ambient air-quality standard sets an 8-h O3 limit of 60 ppb, equivalent to 0.060 ppm or approximately 120 µg/m³ (Taiwan Ministry of Environment, 2024). This standard defines the 60 ppb exceedance threshold applied in the present study. Because O3 formation is strongly modulated by weather and seasonal photochemistry, multi-year monitoring data are essential for distinguishing changes in pollution levels from meteorologically driven variability.

## 1.2 Regional ozone variability in East and Southeast Asia

Surface O3 has become an important air-quality concern across East and Southeast Asia. Regional observation syntheses have shown strong spatial and seasonal variability in surface and tropospheric O3, with high MDA8 O3 and upper-percentile O3 occurring in several densely populated and industrialized regions (Chang et al., 2017; K. Li et al., 2025). Attribution studies further show that both anthropogenic emissions and climate or meteorological variability contribute to observed O3 changes over East and Southeast Asia (Lu et al., 2025). This regional heterogeneity means that local monitoring studies remain necessary even in areas with established air-quality networks, because O3 trends and event risks may differ among cities depending on precursor emissions, meteorology, terrain, and regional circulation.

## 1.3 Meteorological pathways of ozone variability

Meteorological conditions influence O3 through several pathways. High temperature and strong solar radiation accelerate photochemical production, whereas relative humidity, cloudiness, precipitation, and wind fields can suppress or enhance O3 accumulation by modifying radiation, removal processes, atmospheric dilution, and air-mass movement. Weak wind, stagnant circulation, shallow mixing layers, and persistent high-pressure conditions often favor pollutant accumulation, especially at the upper tail of the O3 distribution (Sun et al., 2017). These processes are frequently nonlinear and can vary by season, location, and chemical regime. Generalized additive model studies have reported nonlinear O3 responses to temperature, relative humidity, sunshine duration, wind speed, and co-pollutants (Ma et al., 2020; Hu et al., 2021; Ni et al., 2024), while machine-learning studies have shown that Random Forest and related explainable models can identify nonlinear meteorological drivers of MDA8 or ground-level O3 (Weng et al., 2022; T. Li et al., 2023; Ye et al., 2026).

## 1.4 Taiwan and Kaohsiung context

Taiwan provides a complex setting for O3 pollution because it is affected by monsoonal circulation, tropical systems, coastal circulations, regional inflow, and intensive local emissions. Southern Taiwan is of particular concern because the Kaohsiung-Pingtung region combines dense urban activity, port operations, petrochemical and heavy industries, traffic emissions, coastal meteorology, and terrain effects. Earlier studies identified the Kaoping air basin as an important O3 episode region and showed that mobile sources, industrial emissions, and solvent-related sources were major contributors to atmospheric non-methane organic compounds during O3 episode seasons (Tsai et al., 2004). Photochemical modeling in Kaohsiung further suggested that warm temperature, sufficient sunlight, low wind speed, and high surface pressure are meteorological conditions that tend to trigger O3 episodes in polluted urban environments (Chen et al., 2003).

## 1.5 Mechanistic evidence from southern Taiwan

Several studies have also shown that high O3 in southern Taiwan is not only a surface chemical issue but also a coupled chemistry-circulation problem. Wintertime and autumnal field measurements indicated that O3 can persist overnight in residual layers and be mixed downward to the surface the following morning, contributing to episode development (Lin et al., 2004; Peng et al., 2008). Numerical simulations over southwestern Taiwan showed that autumn O3 episodes are favored by northeasterly flow after frontal passage, blocking by the Central Mountain Range, sunny lee-side conditions, weak winds, and the movement of O3 and precursors from upwind urban and industrial areas (Lin et al., 2007). These studies show why surface O3 observations in Kaohsiung need to be interpreted together with meteorological processes that regulate photochemical production, dilution, vertical mixing, and regional exchange.

## 1.6 Recent Taiwan evidence

More recent studies have extended this understanding by examining local chemical regimes, synoptic influences, and event-scale meteorological drivers. Kuo et al. (2015) used dynamic factor analysis for Kaohsiung and reported that air temperature, relative humidity, sea-land wind conditions, NOx, and CO were associated with hourly O3 variations. Lo et al. (2024) showed that subtropical cyclones and topographic effects can elevate surface O3 before typhoon arrival in southern Taiwan. Lai (2025) further demonstrated that summer O3 pollution events in the Kaoping region were associated with mid-level synoptic patterns, convective stability, humidity and precipitation, solar radiation, and atmospheric dispersion capacity. In addition, recent source-apportionment work using multi-site non-methane hydrocarbon observations in southern Taiwan emphasized the role of petroleum-related, mobile, solvent, aged-air-mass, and regional contributions to O3 formation potential (Nguyen et al., 2026). A recent machine-learning study in northern Taiwan also demonstrated that Random Forest models and SHAP analysis can separate chemical-aerosol and meteorological influences on ground-level O3, identifying temperature, relative humidity, NOx, and VOC species as important predictors (Z. Li et al., 2025). Taken together, these studies show that O3 pollution in Taiwan arises from the interaction of precursor emissions, local photochemistry, vertical mixing, synoptic-scale meteorology, and regional exchange.

## 1.7 Policy-relevant monitoring gap

Despite this body of work, several gaps remain for environmental monitoring and assessment in Kaohsiung. Many previous studies in southern Taiwan focused on selected O3 episodes, specific seasons, vertical profile campaigns, source apportionment, or numerical simulations. The most directly comparable recent study, Lai (2025), characterized the synoptic and meteorological drivers of summer O3 pollution events in the Kaoping region, but focused on warm-season event episodes rather than multi-year, meteorologically adjusted MDA8 O3 trends across the full monitoring network. These studies provide important mechanistic insights, but they do not fully answer a policy-relevant monitoring question: after Taiwan's air-pollution control actions targeting NOx and PM2.5 reductions, has MDA8 O3 in Kaohsiung shown a corresponding response (Taiwan Ministry of Environment, 2023)? Evidence from recent routine monitoring data, especially evidence based on meteorologically adjusted trends, remains limited. A monitoring-based assessment is therefore needed to examine whether recent MDA8 O3 patterns show measurable changes after accounting for weather variability, how high-O3 days are distributed by season and location, and which local meteorological variables are most consistently associated with elevated O3 under routine monitoring conditions.

## 1.8 Methodological rationale

Methodologically, this gap requires both weather-adjusted trend assessment and flexible driver analysis. Because surface O3 has strong seasonal cycles, monthly aggregation with non-parametric trend methods such as the Mann-Kendall or Seasonal Mann-Kendall test and Sen's slope has been used in multi-year ground-level O3 studies (Xu et al., 2016; Xu et al., 2019; Wang et al., 2025). However, observed trends can be obscured or amplified by interannual meteorological variability. Meteorological normalization provides a way to estimate the concentration signal expected under average meteorological conditions, thereby improving the ability to separate air-quality changes from weather-driven fluctuations. Random Forest meteorological normalization has been used for air-pollution trend analysis and for detecting intervention-related changes in air-quality time series (Grange et al., 2018; Grange & Carslaw, 2019). For meteorological drivers, GAMs provide interpretable smooth functions for nonlinear O3-weather relationships, while Random Forest models can capture nonlinear and interacting predictor effects and provide variable-importance measures for explanatory assessment (Hu et al., 2021; Weng et al., 2022; T. Li et al., 2023; Wang et al., 2025). Combining meteorological normalization, seasonal trend tests, GAMs, and Random Forest models is therefore suitable for a monitoring-based study that aims to evaluate recent O3 changes and identify meteorological conditions associated with high-O3 events.

## 1.9 Study objectives

Therefore, this study conducted a monitoring-based assessment of surface O3 in Kaohsiung, Taiwan, from 2018 to 2025. Hourly O3 observations from 12 governmental air-quality monitoring stations were used to calculate MDA8 O3 and examine multi-year, seasonal, monthly, and diurnal variations. Meteorological variables, including temperature, relative humidity, wind speed, wind direction, rainfall, and sunshine duration, were integrated to evaluate the conditions associated with high-O3 days. Seasonal Mann-Kendall tests and Sen's slopes were used to assess the magnitude and temporal pattern of observed and meteorologically normalized MDA8 O3, while GAMs and Random Forest models were applied to quantify nonlinear meteorological effects and identify the relative importance of key drivers. By combining routine monitoring data, quality-control procedures, policy-relevant threshold assessment, trend analysis, meteorological normalization, and interpretable statistical modeling, this study aims to provide evidence for seasonal O3 pollution assessment and air-quality management in an urban-industrial coastal environment.

# References

Chang, K.-L., Petropavlovskikh, I., Cooper, O. R., Schultz, M. G., & Wang, T. (2017). Regional trend analysis of surface ozone observations from monitoring networks in eastern North America, Europe and East Asia. Elementa: Science of the Anthropocene, 5, 50. https://doi.org/10.1525/elementa.243

Chen, K. S., Ho, Y. T., Lai, C. H., & Chou, Y.-M. (2003). Photochemical modeling and analysis of meteorological parameters during ozone episodes in Kaohsiung, Taiwan. Atmospheric Environment, 37, 1811-1823. https://doi.org/10.1016/S1352-2310(03)00079-7

Fleming, Z. L., Doherty, R. M., von Schneidemesser, E., Malley, C. S., Cooper, O. R., Pinto, J. P., Colette, A., Xu, X., Simpson, D., Schultz, M. G., Lefohn, A. S., Hamad, S., Moolla, R., Solberg, S., & Feng, Z. (2018). Tropospheric Ozone Assessment Report: Present-day ozone distribution and trends relevant to human health. Elementa: Science of the Anthropocene, 6, 12. https://doi.org/10.1525/elementa.273

Grange, S. K., Carslaw, D. C., Lewis, A. C., Boleti, E., & Hueglin, C. (2018). Random forest meteorological normalisation models for Swiss PM10 trend analysis. Atmospheric Chemistry and Physics, 18, 6223-6239. https://doi.org/10.5194/acp-18-6223-2018

Grange, S. K., & Carslaw, D. C. (2019). Using meteorological normalisation to detect interventions in air quality time series. Science of the Total Environment, 653, 578-588. https://doi.org/10.1016/j.scitotenv.2018.10.344

Hu, C., Kang, P., Jaffe, D. A., Li, C., Zhang, X., Wu, K., & Zhou, M. (2021). Understanding the impact of meteorology on ozone in 334 cities of China. Atmospheric Environment, 248, 118221. https://doi.org/10.1016/j.atmosenv.2021.118221

Kuo, Y.-M., Chiu, C.-H., & Yu, H.-L. (2015). Influences of ambient air pollutants and meteorological conditions on ozone variations in Kaohsiung, Taiwan. Stochastic Environmental Research and Risk Assessment, 29, 1037-1050. https://doi.org/10.1007/s00477-014-0968-2

Lai, L.-W. (2025). Synoptic and meteorological drivers of summer ozone pollution events in the Kaoping Region, Taiwan. Environmental Monitoring and Assessment, 197, 945. https://doi.org/10.1007/s10661-025-14381-z

Li, K., Tan, R., Qiao, W., Lee, T., Wang, Y., Zhang, D., Tang, M., Zhao, W., Gu, Y., Fan, S., Zhang, J., Lyu, X., Xue, L., Xu, J., Ma, Z., Latif, M. T., Amnuaylojaroen, T., Gil, J., Lee, M.-H., ... Koo, J.-H. (2025). Surface and tropospheric ozone over East Asia and Southeast Asia from observations: Distributions, trends, and variability. Atmospheric Chemistry and Physics, 25, 11575-11596. https://doi.org/10.5194/acp-25-11575-2025

Li, T., Lu, Y., Deng, X., & Zhan, Y. (2023). Spatiotemporal variations in meteorological influences on ambient ozone in China: A machine learning approach. Atmospheric Pollution Research, 14, 101720. https://doi.org/10.1016/j.apr.2023.101720

Li, Z., Wang, Y., Liu, J., & Xian, J. (2025). Using machine learning to unravel chemical and meteorological effects on ground-level ozone: Insights for ozone-climate control strategies. Environment International, 201, 109567. https://doi.org/10.1016/j.envint.2025.109567

Lin, C.-H., Wu, Y.-L., Lai, C.-H., Lin, P.-H., Lai, H.-C., & Lin, P.-L. (2004). Experimental investigation of ozone accumulation overnight during a wintertime ozone episode in south Taiwan. Atmospheric Environment, 38, 4267-4278. https://doi.org/10.1016/j.atmosenv.2004.05.003

Lin, C.-Y., Wang, Z., Chou, C. C.-K., Chang, C.-C., & Liu, S. C. (2007). A numerical study of an autumn high ozone episode over southwestern Taiwan. Atmospheric Environment, 41, 3684-3701. https://doi.org/10.1016/j.atmosenv.2006.12.050

Lo, K.-C., Cheng, W.-H., Lin, C., Hung, C.-H., Yuan, C.-S., & Tseng, Y.-L. (2024). Elevated surface ozone concentration caused by subtropical cyclones and topographical effect: Model simulation and field measurement. Urban Climate, 57, 102093. https://doi.org/10.1016/j.uclim.2024.102093

Lu, X., Liu, Y., Su, J., Weng, X., Ansari, T., Zhang, Y., He, G., Zhu, Y., Wang, H., Zeng, G., Li, J., He, C., Li, S., Amnuaylojaroen, T., Butler, T., Fan, Q., Fan, S., Forster, G. L., Gao, M., ... Nagashima, T. (2025). Tropospheric ozone trends and attributions over East and Southeast Asia in 1995-2019: An integrated assessment using statistical methods, machine learning models, and multiple chemical transport models. Atmospheric Chemistry and Physics, 25, 7991-8028. https://doi.org/10.5194/acp-25-7991-2025

Ma, Y., Ma, B., Jiao, H., Zhang, Y., Xin, J., & Yu, Z. (2020). An analysis of the effects of weather and air pollution on tropospheric ozone using a generalized additive model in western China: Lanzhou, Gansu. Atmospheric Environment, 224, 117342. https://doi.org/10.1016/j.atmosenv.2020.117342

Mills, G., Pleijel, H., Malley, C. S., Sinha, B., Cooper, O. R., Schultz, M. G., Neufeld, H. S., Simpson, D., Sharps, K., Feng, Z., Gerosa, G., Harmens, H., Kobayashi, K., Saxena, P., Paoletti, E., Sinha, V., & Xu, X. (2018). Tropospheric Ozone Assessment Report: Present-day tropospheric ozone distribution and trends relevant to vegetation. Elementa: Science of the Anthropocene, 6, 47. https://doi.org/10.1525/elementa.302

Nguyen, D.-H., Hsieh, H.-C., Liang, M.-C., Lin, N.-H., Wang, C.-H., & Wang, J.-L. (2026). Multi-site non-methane hydrocarbon source apportionment and ozone insights in southern Taiwan using positive matrix factorization. Atmospheric Chemistry and Physics, 26, 2831-2852. https://doi.org/10.5194/acp-26-2831-2026

Ni, J., Jin, J., Wang, Y., Li, B., Wu, Q., Chen, Y., Du, S., Li, Y., & He, C. (2024). Surface ozone in global cities: A synthesis of basic features, exposure risk, and leading meteorological driving factors. Geography and Sustainability, 5, 64-76. https://doi.org/10.1016/j.geosus.2023.09.008

Peng, Y.-P., Chen, K.-S., Lou, J.-C., Hwang, S.-W., Wang, W.-C., Lai, C.-H., & Tsai, M.-Y. (2008). Measurements and mesoscale modeling of autumnal vertical ozone profiles in southern Taiwan. Terrestrial, Atmospheric and Oceanic Sciences, 19, 505-514. https://doi.org/10.3319/TAO.2008.19.5.505(A)

Sun, W., Hess, P., & Liu, C. (2017). The impact of meteorological persistence on the distribution and extremes of ozone. Geophysical Research Letters, 44, 1545-1553. https://doi.org/10.1002/2016GL071731

Taiwan Ministry of Environment. (2024). Air quality standards (amended September 30, 2024). Air Quality Monitoring Network. Retrieved July 17, 2026, from https://airtw.moenv.gov.tw/CHT/Information/Standard/Rules.aspx

Taiwan Ministry of Environment. (2023, November 6). Air pollution control action plan. https://www.moenv.gov.tw/en/news/press-releases/7515.html

Tsai, J.-H., Hsu, Y.-C., & Yang, J.-Y. (2004). The relationship between volatile organic profiles and emission sources in ozone episode region: A case study in southern Taiwan. Science of the Total Environment, 328, 131-142. https://doi.org/10.1016/j.scitotenv.2004.01.020

Wang, J., Dong, J., Li, R., Zhang, X., Xu, Q., & Song, X. (2025). Assessing anthropogenic contributions and uncovering inter-regional periodic patterns of ground ozone with high-resolution predictions in 2015-2019 across China. Science of the Total Environment, 977, 179360. https://doi.org/10.1016/j.scitotenv.2025.179360

Weng, X., Forster, G. L., & Nowack, P. (2022). A machine learning approach to quantify meteorological drivers of ozone pollution in China from 2015 to 2019. Atmospheric Chemistry and Physics, 22, 8385-8402. https://doi.org/10.5194/acp-22-8385-2022

World Health Organization. (2021). WHO global air quality guidelines: Particulate matter (PM2.5 and PM10), ozone, nitrogen dioxide, sulfur dioxide and carbon monoxide. https://www.who.int/publications/i/item/9789240034228

Xu, W., Lin, W., Xu, X., Tang, J., Huang, J., Wu, H., & Zhang, X. (2016). Long-term trends of surface ozone and its influencing factors at the Mt Waliguan GAW station, China - Part 1: Overall trends and characteristics. Atmospheric Chemistry and Physics, 16, 6191-6205. https://doi.org/10.5194/acp-16-6191-2016

Xu, X., Zhang, T., & Su, Y. (2019). Temporal variations and trend of ground-level ozone based on long-term measurements in Windsor, Canada. Atmospheric Chemistry and Physics, 19, 7335-7345. https://doi.org/10.5194/acp-19-7335-2019

Ye, X., Zhang, L., Wang, X., Lu, N., Hickman, S., Luo, G., & Archibald, A. T. (2026). Deciphering the impacts of meteorology on surface ozone variability in eastern China using explainable machine learning models. Atmospheric Chemistry and Physics, 26, 6377-6390. https://doi.org/10.5194/acp-26-6377-2026